Form Energy · Technical Screen Prep

Mechanical Engineer II, Hydrogen System. Fluid Systems team. They’ll probe gas/fluid systems, hydrogen safety, and first-principles reasoning.

1 · THE DOMAIN: why is there hydrogen at all?

▸ Iron-air = reversible rusting in an aqueous alkaline (KOH) electrolyte. Discharge: iron + oxygen + water form iron hydroxide (rust). Charge: current reverses it back to metallic iron and releases oxygen.

▸ The hydrogen comes from a side reaction on charge: the iron electrode sits near the hydrogen-evolution potential in alkaline, so some charging current splits water (2H₂O + 2e⁻ → H₂ + 2OH⁻) instead of reducing iron. That parasitic hydrogen-evolution reaction lowers coulombic efficiency and produces H₂ gas that must be managed.

▸ Safety context: the cell chemistry itself is non-flammable and passed UL9540A with no thermal runaway (a big selling point vs lithium). So the hydrogen system is the key remaining flammability hazard to engineer around.

💬 In the room:  "The H₂ is from the hydrogen-evolution side reaction during charging. It’s a small parasitic fraction, but it’s flammable, so it has to be safely collected, diluted, and vented."

2 · KNOW YOUR FLUID: hydrogen properties that drive design

▸ Smallest molecule → leaks and permeates through tiny gaps, threads, and polymers. Sealing is hard; joints and materials matter more than with other gases.

▸ Wide flammability range: ~4% (LEL) to ~75% (UEL) in air, far wider than most fuels. Keep concentrations below 4%.

▸ Tiny ignition energy (~0.02 mJ): a static spark ignites it. Bond and ground everything; eliminate ignition sources.

▸ Buoyant (14× lighter than air): it rises and collects at high points. Vent and place sensors HIGH.

▸ Colorless, odorless, near-invisible flame: you cannot see or smell a leak. Detection is mandatory.

💬 In the room:  "Design rules follow from the physics: vent high because it’s buoyant, dilute below 4%, kill ignition sources because the ignition energy is tiny, and detect actively because you can’t sense it."

3 · HYDROGEN-SPECIFIC MECHANICAL DESIGN

▸ Hydrogen embrittlement: atomic H diffuses into metals and causes cracking / loss of ductility, worst in high-strength and martensitic steels. Prefer austenitic stainless (304/316L), aluminum, or copper. This is the classic H₂ materials question.

▸ Permeation: H₂ permeates elastomers and polymers over time. Choose seal materials and wall thickness accordingly; expect some loss.

▸ Leak management: minimize joints, prefer welded over threaded, use proper fittings, and He or H₂ leak-check. Design for detectability.

▸ Venting & dilution: route H₂ from high points to a safe outdoor location, dilute with ventilation below the LEL, add flame arrestors, no ignition sources in the path.

▸ Detection: catalytic or electrochemical H₂ sensors at high points, plus pressure-decay monitoring, tied to alarms and interlocks.

💬 In the room:  "For a hydrogen enclosure I’d think: material choice to avoid embrittlement, minimize and weld joints, vent from the top, keep it below 4% with ventilation, sensors up high, and no ignition sources."

4 · GAS & FLUID SYSTEMS FUNDAMENTALS

▸ Pressure drop & flow: flow is driven by ΔP; pressure drop rises with length and flow and falls hard with diameter (Darcy-Weisbach; ΔP ~ 1/d⁵ in the loss term). For gas at high pressure ratios, flow can choke (go sonic) at an orifice.

▸ Components: regulators drop pressure, relief valves protect against overpressure, check valves prevent backflow, flame arrestors stop flashback. Know what each does and why it’s there.

▸ Purging / inerting: use N₂ to purge before and after service so you never pass through a flammable H₂-air mixture.

▸ Leak testing: pressure-decay, bubble test, or tracer-gas (He) leak detection. You did exactly this kind of work on vacuum and gas systems at Zap.

💬 In the room:  "Pressure drop scales roughly with 1 over diameter to the fifth, so if flow resistance is a problem, diameter is the lever, not length."

5 · MECHANICAL FUNDAMENTALS (see the Solid Mechanics sheet)

▸ Expect the classics: stress = F/A, cantilever bending (σ = Mc/I, δ ~ FL³/EI), pressure-vessel hoop stress (σ = pr/t, 2× axial), thermal stress (σ = EαΔT), bolted-joint preload, and fatigue for anything cyclic.

▸ If you blank on a number: say what it depends on and which variable dominates. Reasoning out loud beats a memorized formula.

6 · LIKELY DESIGN PROBLEMS (reason out loud)

▸ "Safely vent H₂ from a battery module." Collect at high points, route outside away from ignition, dilute below 4% with ventilation, flame arrestor, sensors and interlocks. State assumptions and the failure modes.

▸ "Pick a material for hydrogen service." Avoid high-strength steel (embrittlement); use 316L stainless or aluminum; consider permeation for seals.

▸ "Prevent H₂ accumulation in an enclosure." Ventilation and dilution below LEL, vent high, sensors high, no ignition sources, interlock to shut down on alarm.

▸ Approach any of them the same way: clarify requirements, state assumptions, give 2-3 options and trade them, size from first principles, name failure modes, say what you’d test.

7 · YOUR FIT + HONEST GAPS

▸ Lead with: hands-on compressed-gas and high-vacuum operations under human-safety-critical protocols (Zap), first-principles fluid and thermal design, test-rig design/build and validation, materials background, and battery context from MarineSitu.

▸ Name the gap plainly: "I haven’t designed a productized hydrogen system to codes like NFPA 2, and my prototyping is research-scale, not high-volume manufacturing. The gas-handling, safety, and first-principles foundation transfers directly, and I ramp fast." No apologizing.

8 · QUESTIONS TO ASK THEM

▸ What stage is the hydrogen system at, prototype or production ramp, and what would I own first?

▸ How do you balance moving fast with the safety demands of a hydrogen system?

▸ What codes and standards govern the design (NFPA 2, ventilation requirements)?

▸ What’s the hardest open problem on the hydrogen system right now?

Night before: why H₂ forms (charging side reaction) · the four H₂ hazards (buoyant, wide flammability 4-75%, tiny ignition energy, invisible) · embrittlement → use 316L/aluminum · vent high + dilute below 4% + detect + no ignition · ΔP ~ 1/d⁵ · hoop stress pr/t · your Zap gas/vacuum safety story. Review the Solid Mechanics sheet.