Solid Mechanics 101 — Interview Cheat Sheet

The 15-ish things that actually get asked. Reason out loud, name the variable that dominates, and you win.

🧱  1 · STRESS & STRAIN (the atoms of everything)

Stress  σ = F / A        Strain  ε = ΔL / L        Hooke:  σ = Eε

▸ Stress = force spread over area. Strain = fractional stretch. E (Young’s modulus) is stiffness: steel ~200 GPa, aluminum ~70 GPa.

▸ Normal stress (pull/push) vs shear stress (slide). Yield = permanent; fracture = broken.

💬 In the room:  "Stress is F over A, so to drop stress I add area or lower load." Simple, correct, confident.

📏  2 · CANTILEVER BEAM (the one that got me!)

Bending stress  σ = M c / I        Tip deflection  δ = F L³ / (3 E I)

▸ M = bending moment (F × length), c = distance to outer fiber, I = area moment of inertia (resistance to bending).

▸ Rectangle: I = b h³ / 12. Height is CUBED — depth matters far more than width.

▸ Deflection scales with L³: double the length, 8× the sag. This is the intuition they want.

💬 In the room:  "Stiffen a cantilever? Add depth (h³), add ribs, or shorten span — not just thicken it."

🌀  3 · SPRINGS

Hooke  F = k x        Series 1/k = 1/k₁ + 1/k₂        Parallel k = k₁ + k₂

▸ k = stiffness. Springs in parallel add up (stiffer); in series get softer (like resistors, but flipped).

▸ Energy stored  U = ½ k x². A beam is really just a spring: k = 3EI/L³ for a cantilever tip.

💬 In the room:  "A cantilever’s stiffness is 3EI over L-cubed — so a beam and a spring are the same idea."

💥  4 · BUCKLING (slender things fail early)

Euler critical load  P_cr = π² E I / (K L)²

▸ Long thin columns fail by buckling (sideways collapse) well before the material yields.

▸ Depends on L² and I, not strength. K = end condition (pinned 1.0, fixed-fixed 0.5, cantilever 2.0).

💬 In the room:  "A vacuum vessel or long strut is a buckling problem, not a strength problem — add stiffening rings."

🔁  5 · FATIGUE (death by a thousand cycles)

S-N curve · endurance limit · stress concentration K_t

▸ Parts fail under repeated loads far below their static strength. Pulsed machines live here.

▸ Steel has an endurance limit (safe forever below it); aluminum does NOT. Sharp corners = stress risers = cracks start there.

💬 In the room:  "For cyclic loads I design below the endurance limit and kill stress concentrations — fillets, good surface finish."

🔩  6 · BOLTED JOINTS & PRELOAD

Preload  F = T / (K d)     (K ≈ 0.2 for dry steel)

▸ A tightened bolt is pre-stretched. External load mostly rides on top of that preload, so the bolt barely feels the cycling — huge fatigue win.

▸ Preload also stops the joint separating and loosening. Torque sets preload via the nut factor K.

💬 In the room:  "Preload is why bolted joints survive vibration — the bolt sees a small fluctuation on a big mean load."

🌡️  7 · THERMAL STRESS (heat = force)

Free expansion  ΔL = α L ΔT        Constrained stress  σ = E α ΔT

▸ Heat something and it grows by αΔT. If it can’t grow, that strain becomes stress — and E is large, so stress gets big fast.

▸ Why designs need expansion joints, slotted holes, and flexures. Critical in pulsed/rapid-heating hardware.

💬 In the room:  "Constrain thermal expansion and you get EαΔT of stress — so I’d add a flexure or slot to let it breathe."

🛢️  8 · PRESSURE VESSELS & TORSION (quick hits)

Hoop stress  σ = p r / t   (twice the axial)      Torsion  τ = T r / J

▸ Thin-wall pressure: hoop stress is 2× axial — that’s why pipes split along their length. Vacuum vessels fail by buckling inward (external pressure).

▸ Torsion twists shafts; J is the polar moment of inertia (torsion’s version of I).

⭐ THE ONE MOVE THAT WORKS EVERY TIME:  don’t panic for the exact number. Say what it depends on and which variable dominates (h³, L³, L², EαΔT). Reasoning out loud from the physics beats a memorized formula every time.