Solid Mechanics 101 — Illustrated

Reason out loud, name the variable that dominates, and you win. Numbers optional; intuition is the point.

📏  1 · CANTILEVER BEAM  (the one that got me!)

σ = M c / I        δ = F L³ / (3 E I)

▸ I = b h³ / 12: height is CUBED, so depth beats width. Deflection scales with L³.

💬 "Stiffen it? Add depth or ribs, or shorten the span. Length dominates: 2× longer = 8× the sag."

🌀  2 · SPRINGS  (a beam is just a spring)

F = k x        U = ½ k x²        cantilever tip: k = 3EI / L³

▸ Parallel adds (stiffer); series softens. Same math as a beam — k = 3EI/L³.

💥  3 · BUCKLING  (slender = collapses early)

P_cr = π² E I / (K L)²

▸ Long thin things fail by buckling before yielding. Depends on L² and I, not strength. K: pinned 1.0, fixed 0.5, cantilever 2.0.

💬 "A vacuum vessel or long strut is a buckling problem — add stiffening rings, don’t just add strength."

🔩  4 · BOLTED JOINTS & PRELOAD

Preload  F = T / (K d)     (K ≈ 0.2 dry steel)

▸ A pre-stretched bolt makes the cyclic load a tiny wiggle on a big mean — huge fatigue win, and it stops loosening.

🌡️  5 · THERMAL STRESS  (heat = force)

ΔL = α L ΔT        constrained:  σ = E α ΔT

▸ Block the growth and strain becomes stress; E is big, so it climbs fast. Add flexures / slots to let it breathe.

🧱  6 · THE REST, IN ONE LINE EACH

▸ Stress/strain: σ = F/A, ε = ΔL/L, σ = Eε. E = stiffness (steel ~200 GPa, Al ~70).

▸ Fatigue: repeated loads fail below static strength; steel has an endurance limit, aluminum does not; kill stress risers with fillets.

▸ Pressure vessel: hoop stress σ = pr/t is 2× axial; vacuum vessels buckle inward.

⭐ THE MOVE: blanked on the number? Say what it depends on and which variable dominates (h³, L³, L², EαΔT). "Deflection goes as L-cubed over EI, so length matters most" beats a memorized constant.