Heat Transfer & Fluids · 10 Conceptual Problems

Answer out loud before reading. The goal is not the number, it’s naming the mechanism and what dominates.

1.  How does a spacecraft reject heat with no air or water?

Think:  no medium means no convection and no conduction to surroundings. The only path out is thermal radiation to space.

Technical:  Q = εσA(T⁴ − T_env⁴). To reject more: more area, higher emissivity, or higher radiator temperature, and temperature dominates because of the T⁴.

💬 "In vacuum it’s radiation-limited, so I’d run the radiator as hot as the electronics allow and maximize εA, because rejection scales with T to the fourth."

2.  A thick steel block and a thin steel sheet, same material, cool in air. Which has a uniform internal temperature?

Think:  the thin one. If heat conducts through the part faster than the surface sheds it, the inside stays uniform.

Technical:  Biot number Bi = hL/k. Bi << 1 means internal conduction wins, temperature is uniform (lumped). Bi >> 1 means the interior lags and gradients form.

💬 "That’s a Biot-number question, small Bi means I can treat it as lumped; large Bi means I have to resolve the internal gradient."

3.  Why do heat sinks have fins, and when does adding more fins stop helping?

Think:  fins add surface area to help a weak convective side (air). But a long thin fin can’t conduct heat to its tip, and cramming fins chokes the airflow between them.

Technical:  fins help most when h is low and k is high (small Bi). Fin efficiency falls as fins get long/thin; too many fins add base resistance and throttle the boundary-layer flow.

💬 "Fins trade conduction for convective area, so they pay off when the air side is the bottleneck, up to the point the fin tips go cold or the channels choke."

4.  Why does boiling remove so much more heat than just heating a liquid?

Think:  a phase change soaks up enormous energy at nearly constant temperature, and the bubbling violently stirs the surface.

Technical:  latent heat is huge (water ~2257 kJ/kg) and boiling HTC is very high. So two-phase moves far more heat per unit area and per unit flow than single-phase, at near-constant temp.

💬 "Two-phase cooling wins on both latent heat and a big heat-transfer coefficient, which is why cold plates go two-phase for high flux."

5.  What is critical heat flux, and why does it set the limit on a cold plate?

Think:  push a boiling surface too hard and a vapor film blankets it. Vapor is a terrible conductor, so the surface can’t rewet, and temperature spikes, burnout.

Technical:  CHF is the flux where nucleate boiling transitions to film boiling (dryout); HTC collapses and ΔT jumps. Design lives at a margin below CHF.

💬 "CHF is the dryout cliff. My UCSD work characterized wicks up to 600 W/cm², and margin to CHF is the whole game."

6.  You need more coolant flow but pressure drop is too high. Bigger diameter or shorter tube?

Think:  diameter, by a lot. Pressure drop is extremely sensitive to how wide the pipe is.

Technical:  laminar (Hagen-Poiseuille) ΔP ∝ 1/d⁴ at fixed flow; turbulent even stronger (~1/d⁵ for fixed mass flow). Length is only linear. Diameter is the lever.

💬 "Pressure drop goes as roughly 1 over diameter to the fourth or fifth, so I’d open up the diameter before shortening the run."

7.  Two machined metal blocks are bolted face to face. Why do they conduct heat poorly across the joint?

Think:  "flat" surfaces really touch only at tiny high spots; most of the interface is air gaps, and air barely conducts.

Technical:  thermal contact resistance: real contact is a few percent of area at asperities. Fix with a thermal interface material and higher clamping pressure to grow real contact area.

💬 "That’s contact resistance, they only touch at asperities. I’d add a TIM and clamp harder to increase the true contact area."

8.  You heat one end of a copper rod and one end of a plastic rod. Why does copper get hot at the far end so much faster?

Think:  copper spreads heat quickly; plastic hoards it. It’s about how fast temperature diffuses, not just conductivity.

Technical:  thermal diffusivity α = k/(ρc_p). Penetration time t ~ L²/α (Fourier number Fo = αt/L²). High-α copper heats through fast; low-α plastic lags.

💬 "Transient response is set by diffusivity, and time scales with length squared over α, so thin high-α parts respond fast."

9.  Why does turbulent flow transfer heat better than laminar, and what does it cost?

Think:  turbulence mixes the fluid and thins the stagnant layer at the wall where the temperature drop hides, but churning the fluid costs pressure.

Technical:  Re = ρvd/μ (inertia/viscous). High Re thins the thermal boundary layer, raising Nu and h; the price is higher ΔP. Nu ties Re and Pr to heat transfer.

💬 "Heat transfer lives in the near-wall boundary layer; turbulence thins it, so h goes up, at the cost of pumping power."

10.  Gas vents through a small orifice. You drop the downstream pressure more, but flow stops increasing. Why?

Think:  once the gas hits the speed of sound at the throat, downstream changes can’t travel back upstream, so the flow can’t "hear" that you lowered the pressure.

Technical:  choked flow: below the critical pressure ratio (~0.53 for air), the throat is sonic and mass flow depends only on upstream conditions. Key for feed systems, relief, and venting.

💬 "That’s choked flow, at sonic throat conditions mass flow is set by upstream state alone, which is how you’d size a relief or vent."

The move every time: name the mechanism (radiation? boundary layer? phase change? choking?), then the variable that dominates (T⁴, 1/d⁴or⁵, latent heat, L²/α). Reasoning out loud beats a memorized number.