MATH FOUNDATIONS

for Mechanical Engineers

Reference Sheet & Problem Set

Designed for the returning engineer.

No shame. No rush. Just rebuilding fluency.

How to use this document:

Each section has: Key Formulas → Worked Example → Problems (3 levels)

Do 2–3 problems per sitting. Walk away when you feel the wall.

Warm-Ups rebuild confidence. Stretches rebuild skill. Challenges rebuild depth.

# 1. Linear Algebra

Why this matters for MechE: Stiffness matrices in FEA, principal stresses via eigenvalues, coordinate transformations, vibration mode shapes. This is the language your FEA solver speaks.

## 1.1 Matrix Operations

| Matrix multiply | (AB)_ij = Σ_k A_ik B_kj |
| --- | --- |
| Transpose | (A^T)_ij = A_ji |
| Inverse (2x2) | A⁻¹ = (1/det) [d, -b; -c, a] |
| Determinant (2x2) | det(A) = ad - bc |
| Determinant (3x3) | Cofactor expansion along any row/col |

Worked Example

Find the inverse of A = [3, 1; 5, 2]

det(A) = (3)(2) - (1)(5) = 6 - 5 = 1

A⁻¹ = (1/1) [2, -1; -5, 3] = [2, -1; -5, 3]

Check: AA⁻¹ = [3(2)+1(-5), 3(-1)+1(3); 5(2)+2(-5), 5(-1)+2(3)] = [1,0; 0,1] ✓

Warm-Up Problems

1. Multiply A = [1,2; 3,4] by B = [5,6; 7,8]. Verify (AB)^T = B^T A^T.

2. Find det(A) for A = [2,3,1; 0,1,4; 5,2,0]. Use cofactor expansion along the first row.

3. If A is 3x2 and B is 2x4, what size is AB? Can you compute BA?

Stretch Problems

1. Find A⁻¹ for A = [4,7; 2,6]. Verify by computing AA⁻¹.

2. Show that for any square matrix, det(A^T) = det(A). Try it with a specific 3x3.

3. A rotation matrix R(θ) = [cosθ, -sinθ; sinθ, cosθ]. Show that R⁻¹ = R^T (i.e., R is orthogonal).

Challenge Problems

1. For A = [1,2; 3,4], compute A² - 5A - 2I. What do you notice? (Hint: Cayley-Hamilton theorem.)

2. Prove that if det(A) = 0, the system Ax = b either has no solution or infinitely many. Give a geometric interpretation for 2x2.

## 1.2 Eigenvalues & Eigenvectors

Why this matters for MechE: Principal stresses are eigenvalues of the stress tensor. Natural frequencies are eigenvalues of the system matrix. Mode shapes are eigenvectors. This is not abstract — it is the core of structural analysis.

| Eigenvalue equation | Av = λv → (A - λI)v = 0 |
| --- | --- |
| Characteristic eqn | det(A - λI) = 0 |
| Trace | λ₁ + λ₂ + ... = tr(A) = Σ a_ii |
| Determinant | λ₁ · λ₂ · ... = det(A) |

Worked Example

Find eigenvalues of A = [4, 2; 1, 3]

det(A - λI) = (4-λ)(3-λ) - 2 = λ² - 7λ + 10 = (λ-5)(λ-2) = 0

λ₁ = 5, λ₂ = 2

For λ₁=5: (A-5I)v = [-1,2;1,-2]v = 0 → v₁ = [2,1]^T

For λ₂=2: (A-2I)v = [2,2;1,1]v = 0 → v₂ = [1,-1]^T

Check: tr(A) = 7 = 5+2 ✓, det(A) = 10 = 5×2 ✓

Warm-Up Problems

1. Find eigenvalues of [6, -1; 2, 3]. Use trace/det to verify.

2. Find the eigenvectors for [5, 4; 1, 2].

3. If A has eigenvalues 3 and 7, what are the eigenvalues of A + 2I? Of 3A?

Stretch Problems

1. Find eigenvalues and eigenvectors of the symmetric matrix [2,1;1,2]. What angle do the eigenvectors make? Why are they orthogonal?

2. A stress tensor in 2D is σ = [50, 30; 30, -20] MPa. Find the principal stresses (eigenvalues) and principal directions (eigenvectors).

Challenge Problems

1. The stiffness matrix for a 2-DOF spring-mass system is K = [3,-1;-1,2] and mass matrix M = [2,0;0,1]. Find natural frequencies from det(K - ω²M) = 0.

Hint: The eigenvalue problem Kv = ω²Mv is a generalized eigenvalue problem. Substitute μ = ω² and solve the characteristic equation.

# 2. Calculus & Vector Calculus

Why this matters for MechE: Gradients drive heat flow. Divergence appears in conservation laws. Integrals give you resultant forces and moments. The divergence theorem connects volume integrals to surface integrals — the foundation of every finite element formulation.

## 2.1 Multivariable Calculus

| Gradient | ∇f = (∂f/∂x)î + (∂f/∂y)ĵ + (∂f/∂z)k̂ |
| --- | --- |
| Divergence | ∇·F = ∂F₁/∂x + ∂F₂/∂y + ∂F₃/∂z |
| Curl | ∇×F = det[i,j,k; ∂/∂x,∂/∂y,∂/∂z; F₁,F₂,F₃] |
| Laplacian | ∇²f = ∂²f/∂x² + ∂²f/∂y² + ∂²f/∂z² |
| Chain rule | df/dt = (∂f/∂x)(dx/dt) + (∂f/∂y)(dy/dt) + ... |
| Divergence theorem | ∫∫∫ ∇·F dV = ∮ F·n̂ dA |

Worked Example

Given F = (x²y, xz, y²z), find ∇·F

∂(x²y)/∂x = 2xy

∂(xz)/∂y = 0

∂(y²z)/∂z = y²

∇·F = 2xy + 0 + y² = 2xy + y²

Warm-Up Problems

1. Find the gradient of f(x,y) = x² + 3xy - y². Evaluate at (1,2).

2. Compute ∇·F for F = (3x, -2y, z). What does a constant divergence mean physically?

3. Find all partial derivatives of f(x,y,z) = xe^(yz).

Stretch Problems

1. The temperature field is T(x,y) = 100 - x² - 4y². Find ∇T at (1,1). In which direction does temperature decrease fastest?

2. Verify that ∇×(∇f) = 0 for f(x,y,z) = x²y + yz².

3. Compute the Laplacian ∇²T for T = 100 - x² - 4y². If ∇²T < 0, what does that tell you about heat flow at that point (no sources)?

Challenge Problems

1. Use the divergence theorem: evaluate ∮ F·n̂ dA where F = (x,y,z) over the surface of a unit sphere centered at the origin. (Compute the volume integral instead.)

2. The equilibrium equation in solid mechanics is ∇·σ + b = 0. If σ_xx = 3x²y, σ_xy = -x³/3 + g(y), what must g(y) satisfy for equilibrium with zero body forces?

# 3. Ordinary Differential Equations

Why this matters for MechE: Vibration of a spring-mass-damper = 2nd order ODE. Transient heat conduction (lumped system) = 1st order. Control system response = system of ODEs. If you can solve ODEs by hand, you understand what your simulation software is doing.

## 3.1 First-Order ODEs

| Separable | dy/dx = f(x)g(y) → ∫dy/g(y) = ∫f(x)dx |
| --- | --- |
| Linear (1st order) | y' + P(x)y = Q(x) |
| Integrating factor | μ = e^(∫P(x)dx) |
| Solution | y = (1/μ)∫μQ(x)dx |
| Exponential decay | y' = -ky → y = y₀ e^(-kt) |

Worked Example

Solve y' + 2y = 6, with y(0) = 1

Integrating factor: μ = e^(∫2dx) = e^(2x)

Multiply through: (e^(2x) y)' = 6e^(2x)

Integrate: e^(2x) y = 3e^(2x) + C

y = 3 + Ce^(-2x). Apply IC: 1 = 3 + C → C = -2

y(x) = 3 - 2e^(-2x)

Sanity check: as x→∞, y→3 (steady state). At x=0, y=1 ✓

Warm-Up Problems

1. Solve dy/dx = 3x², y(0) = 5. (Direct integration.)

2. Solve the separable equation dy/dx = xy, y(0) = 2.

3. A hot part cools according to T' = -0.1(T - 25), T(0) = 200°C. Find T(t) and the time to reach 50°C.

Stretch Problems

1. Solve y' + y/x = x, for x > 0. (Hint: integrating factor is x.)

2. A tank has 100L of water with 10kg of salt. Pure water enters at 5L/min, well-mixed solution exits at 5L/min. Find salt amount S(t).

## 3.2 Second-Order Linear ODEs

Why this matters for MechE: my'' + cy' + ky = F(t) is the vibration equation. Every mechanical engineer should be able to write and solve this in their sleep.

| General form | ay'' + by' + cy = f(t) |
| --- | --- |
| Characteristic eqn | ar² + br + c = 0 |
| Overdamped | Two real roots: y = C₁e^(r₁t) + C₂e^(r₂t) |
| Critically damped | Repeated root: y = (C₁ + C₂t)e^(rt) |
| Underdamped | Complex roots: y = e^(αt)(C₁cosωt + C₂sinωt) |
| Natural frequency | ω_n = √(k/m) |
| Damping ratio | ζ = c / (2√(mk)) |

Worked Example

Solve y'' + 4y' + 3y = 0, y(0) = 2, y'(0) = -4

Characteristic equation: r² + 4r + 3 = (r+1)(r+3) = 0

r₁ = -1, r₂ = -3 (overdamped)

y = C₁e^(-t) + C₂e^(-3t)

IC1: y(0) = C₁ + C₂ = 2

IC2: y'(0) = -C₁ - 3C₂ = -4 → C₁ = 1, C₂ = 1

y(t) = e^(-t) + e^(-3t)

Warm-Up Problems

1. Classify without solving: y'' + 6y' + 9y = 0 (find discriminant of characteristic equation).

2. Solve y'' + 9y = 0, y(0) = 0, y'(0) = 3. What is the natural frequency?

3. A 2kg mass on a spring (k = 200 N/m) with no damping. Write the equation of motion and find ω_n.

Stretch Problems

1. Solve y'' + 2y' + 5y = 0, y(0) = 1, y'(0) = 0. Identify α, ω_d, and sketch the response.

2. A spring-mass-damper: m=1, c=4, k=3. Is it overdamped, critically damped, or underdamped? Find y(t) for y(0)=1, y'(0)=0.

3. For the system my'' + cy' + ky = F₀cos(ωt), what happens at resonance (ω = ω_n) with no damping?

Challenge Problems

1. Solve y'' + 4y = 3cos(2t) using undetermined coefficients. Why does the standard guess fail? What modification is needed?

2. A machine (50 kg) is on a mount (k=20000 N/m, c=500 Ns/m). It experiences F=100sin(40t) N. Find the steady-state amplitude. Is this near resonance?

# 4. Partial Differential Equations (Intro)

Why this matters for MechE: The heat equation, wave equation, and equilibrium equation are all PDEs. FEA is a method for solving PDEs numerically. Understanding the analytical forms helps you set up boundary conditions correctly and interpret results.

## 4.1 Key Equations

| Heat equation | ∂T/∂t = α ∇²T |
| --- | --- |
| Wave equation | ∂²u/∂t² = c² ∇²u |
| Laplace equation | ∇²φ = 0 (steady state, no sources) |
| Poisson equation | ∇²φ = f(x,y,z) (with sources) |
| Separation of vars | u(x,t) = X(x)T(t) |

Worked Example

Heat equation in 1D: ∂T/∂t = α ∂²T/∂x² on a rod [0,L], with T(0,t) = T(L,t) = 0

Assume T(x,t) = X(x)·T(t). Substituting and separating:

T'/(αT) = X''/X = -λ (both sides must equal a constant)

X'' + λX = 0 with X(0) = X(L) = 0 → X_n = sin(nπx/L), λ_n = (nπ/L)²

T' + αλ_n T = 0 → T_n = e^(-α(nπ/L)² t)

Solution: T(x,t) = Σ B_n sin(nπx/L) e^(-α(nπ/L)² t)

Warm-Up Problems

1. For the heat equation ∂T/∂t = 3∂²T/∂x², what is α? What are its units if x is in meters and t in seconds?

2. Classify each as elliptic, parabolic, or hyperbolic: (a) ∇²u = 0, (b) u_t = u_xx, (c) u_tt = c²u_xx.

3. If T(x,t) = sin(πx/L) e^(-απ²t/L²), verify it satisfies the heat equation.

Stretch Problems

1. A rod of length 1m, α = 0.01 m²/s, initial temp T(x,0) = sin(πx). Ends held at 0°C. Write the solution and find T(0.5, 10).

2. For the vibrating string u_tt = 4u_xx, u(0,t) = u(π,t) = 0, find the first three mode shapes and frequencies.

Challenge Problems

1. The Euler-Bernoulli beam equation is EI ∂⁴w/∂x⁴ = q(x) for static loading. For a simply supported beam with uniform load q, integrate four times applying BCs: w(0) = w(L) = 0, w''(0) = w''(L) = 0. Find w(x).

# 5. Tensor & Index Notation

Why this matters for MechE: Stress, strain, stiffness — all tensors. Index notation lets you write complex relationships compactly and is essential for reading continuum mechanics and FEA literature. If you can read index notation, you can read any solid mechanics paper.

## 5.1 Key Notation & Identities

| Einstein summation | σ_ij = C_ijkl ε_kl (sum over repeated indices) |
| --- | --- |
| Kronecker delta | δ_ij = 1 if i=j, 0 if i≠j |
| Stress tensor | σ = [σ_xx, σ_xy, σ_xz; ...] (symmetric: σ_ij = σ_ji) |
| Strain tensor | ε_ij = (1/2)(u_i,j + u_j,i) |
| Hooke’s law (iso) | σ_ij = λ ε_kk δ_ij + 2μ ε_ij |
| Von Mises stress | σ_vm = √(3/2 s_ij s_ij), s_ij = σ_ij - (1/3)σ_kkδ_ij |

Worked Example

Expand ε_kk (trace of strain tensor):

ε_kk = ε_11 + ε_22 + ε_33 = ε_xx + ε_yy + ε_zz

This is the volumetric strain (dilatation) — the relative change in volume.

Expand Hooke’s law for σ_12:

σ_12 = λ ε_kk δ_12 + 2μ ε_12

Since δ_12 = 0: σ_12 = 2μ ε_12 = μ γ_12 (shear stress = G × engineering shear strain)

Warm-Up Problems

1. Expand σ_ij δ_ij using the summation convention. What is this quantity physically?

2. Write out all 4 terms of σ_ij n_j for i=1 in 2D (j runs 1,2). What does this represent?

3. Given ε = [0.002, 0.001; 0.001, -0.001], find the volumetric strain.

Stretch Problems

1. For plane stress (σ_33 = σ_13 = σ_23 = 0), simplify Hooke’s law to get σ_11 in terms of ε_11 and ε_22 only (eliminate ε_33 using σ_33 = 0).

2. Given σ = [100, 40, 0; 40, -50, 0; 0, 0, 30] MPa, compute the deviatoric stress tensor s_ij and the von Mises stress.

Challenge Problems

1. Starting from σ_ij = λ ε_kk δ_ij + 2μ ε_ij, invert to get ε_ij in terms of σ_ij. Express the result using E and ν instead of λ and μ.

Hint: Take the trace of both sides first to express ε_kk in terms of σ_kk. Then substitute back. Use λ = Eν/((1+ν)(1-2ν)) and μ = E/(2(1+ν)).

# Selected Answers & Checkpoints

These aren’t full solutions — they’re checkpoints so you know if you’re on the right track. If you get stuck, the goal is to identify exactly where you’re stuck, not to push through confusion.

## Section 1: Linear Algebra

1.1 Warm-Up 1: AB = [19,22; 43,50]

1.1 Warm-Up 2: det(A) = -37

1.1 Stretch 1: A⁻¹ = (1/10)[6,-7; -2,4]

1.2 Warm-Up 1: λ = 5, 4

1.2 Stretch 2: Principal stresses ≈ 59.1 MPa and -29.1 MPa

## Section 2: Vector Calculus

2.1 Warm-Up 1: ∇f = (2x+3y, 3x-2y). At (1,2): (8, -1)

2.1 Warm-Up 2: ∇·F = 3-2+1 = 2 (constant → uniform source/sink everywhere)

2.1 Challenge 1: ∇·F = 3, so ∫∫∫ 3 dV = 3 × (4π/3) = 4π

## Section 3: ODEs

3.1 Warm-Up 2: y = 2e^(x²/2)

3.1 Warm-Up 3: T(t) = 25 + 175e^(-0.1t). Time to 50°C ≈ 25.5 s (actually, check: ln(175/25)/0.1 ≈ 19.5 s)

3.2 Warm-Up 2: y = sin(3t). ω_n = 3 rad/s

3.2 Warm-Up 3: 2y'' + 200y = 0 → ω_n = 10 rad/s

## Section 4: PDEs

4.1 Warm-Up 1: α = 3, units m²/s (thermal diffusivity)

4.1 Warm-Up 2: (a) elliptic, (b) parabolic, (c) hyperbolic

## Section 5: Tensors

5.1 Warm-Up 1: σ_ijδ_ij = σ_ii = σ_11+σ_22+σ_33 (trace = 3 × hydrostatic stress)

5.1 Warm-Up 3: ε_vol = 0.002 + (-0.001) = 0.001

5.1 Stretch 2: Hydrostatic = 80/3 MPa. Compute s_ij, then σ_vm ≈ 132.3 MPa