# Meeting Agenda

IMTA Data Check with Corey

Aditi Kalle  |  DOE/ORISE Marine Energy Fellow  |  August 17, 2026

## The ask

I've been analyzing the IMTA buoy telemetry (post-retrieval): solar harvest, battery, panel behavior, and building forecasting and scheduling models on it.

You know the IMTA system far better than I do. I want to walk you through the trends I'm seeing and get you to confirm or correct them against how the system actually behaves, so I'm not building conclusions on a sensor artifact.

## What I've done (quick)

- Pulled the per-minute telemetry (panel power, batt voltage/current, yield, state)

- Cleaned it to the May 20 to June 24 window

- Built a panel-health monitor (compares output to satellite irradiance), a battery/SoC model, and solar forecasting + a camera scheduler

## What I need you to verify

### 1. The ~60% panel-output drop from early June (the big one)

My health monitor flags the panel producing far less than the sunshine warrants, starting ~June 4 and worsening.

- Given what you know about IMTA deployments, is this biofouling, shading, or tilt?

- Or would you suspect wiring, charge controller, or the panel_w sensor?

- Was the panel inspected after retrieval? Photos?

### 2. June 25: zero panel output all day

0 W the whole day, but data still flowing.

- On IMTA, does that read as a sensor flatline or a real panel failure?

### 3. Battery / SoC

- I'm recommending coulomb counting (voltage-only SoC is ~15% off in the flat LiFePO4 band).

- Does the IMTA BMS log current so I can do that, or is voltage all that's stored?

- Confirm: 1200 Wh usable, 20% floor. The right assumption for IMTA?

### 4. The state field and switching

- What do the state codes mean on IMTA (running / sleep / fault)?

- Do the cameras and sensors switch synchronized or independently?

### 5. Sanity-check my numbers

- ~230 Wh/day harvested, ~1.7 W base draw, ~34 W camera.

- Do those line up with the IMTA hardware (3× C3 cameras, Jetson NX, cellular)?

## Outcomes I want

- ☐  June 4 drop: physical (fouling) vs. sensor/wiring?

- ☐  June 25 zero: sensor vs. real?

- ☐  Current logging: available for coulomb counting?

- ☐  State codes + sync vs. independent switching

- ☐  Power numbers in the right range for IMTA?