# IMTA Power Optimization

Monday Check-In

July 28, 2026  |  Mitchell Scott, Paul Murphy, Aditi Kalle

## 1. Optimization Framework

### Architecture

Telemetry (35 days) → LSTM Solar Forecast → Proportional Allocation → Daily Schedule

24h lookback          by priority              duty cycle %

Three approaches tested:

- CVXPY Integer LP (48h binary on/off): maximizes uptime window, unrealistic for deployment

- Proportional Allocation (daily duty cycle): sustainable, selected for use

- Seasonal Model (week+ planning): long-term backup

Figure: how the 9.6 W average splits into 1.7 W (base load) and 7.9 W (camera duty). Base load takes its share first; camera adapts.

## 2. Deployment Parameters

| Parameter | Value |
| --- | --- |
| Location | Gulf Coast AL (30.2°N, 87.965°W) |
| Battery | 12V 100Ah LiFePO4 = 1200 Wh |
| Solar Panel | 100W rated |
| Safe SoC Range | 20 to 90% (240 to 1080 Wh) |
| Data Window | May 20 to June 24, 2026 (35 days, 49K records) |

System allocation by priority:

- Base Load (Jetson + Router): 1.7 W always-on

- Camera (PoE): 33.7 W, duty-cycle controlled

## 3. Model Comparison

### LSTM vs Prophet (Head-to-Head)

| Metric | LSTM | Prophet | Winner |
| --- | --- | --- | --- |
| MAE | 0.93 W | 3.58 W | LSTM (4× better) |
| Generalization | Train 0.63 W ≈ Test 0.93 W | Trend-based, assumes patterns | LSTM |
| Data scaling | Stabilizes at 41 days | High MAE on small datasets | LSTM |
| Intra-day swings | Yes (24h lookback) | No (smooths) | LSTM |
| Needs weather data | No | No | Tie |

Why LSTM wins on IMTA:

- 24-hour lookback captures the daily solar cycle without weather input

- Small dataset (41 days): temporal patterns beat statistical trends

- High cloud and wind volatility at sea: LSTM's minute-level reactivity beats smoothing

Why Prophet might fail:

- Assumes repeating seasonal patterns (ocean weather is random)

- Multiplicative seasonality assumes stable baseline (not true at sea)

- Smoothing removes dynamics needed for hourly scheduling

Recommendation: Use LSTM for hourly and daily scheduling. Prophet for 7+ day planning only.

## 4. MAE vs Power Allocation: Fractional Analysis

### The Real Constraint

Average daily solar:   230 Wh/day

Average hourly power:  9.6 W

LSTM forecast error:   ±0.93 W  (9.7% of available solar)

Camera power draw:     33.7 W   (3.5× average solar)

Base load:             1.7 W    (always on)

Usable camera duty:    (9.6 - 1.7) / 33.7 = 23% of day

### Deployment Scenarios

Good Day (40 W avg solar):

- Camera duty: 38.3 W available / 33.7 W draw = 100% uptime

- LSTM ±0.93 W error → 97-99% achievable ✓

Bad Day (0.9 W avg solar):

- Problem: Base load (1.7 W) > solar (0.9 W)

- Must discharge battery → camera = 0%

- Real SoC monitoring required, not forecast-only ✗

### Sustainable Strategy

If LSTM < (1.7 + 5 W safety margin):

→ Camera off, rely on battery reserve

If LSTM > (1.7 + 20 W):

→ Run camera at optimized duty cycle

Else:

→ Real-time battery SoC monitoring, adjust per hour

## 5. Benchmark Findings (41-Day Dataset)

Why most models fail:

- AutoARIMA (1.80 W): statistical assumptions fail on sea-based data

- Linear models (~4 W): can't learn nonlinear temporal patterns

- Tree ensembles: mixed. RF unstable, GB reasonable at 0.39 W

Why LSTM survives:

- Handles small datasets better than statistical methods

- Learns patterns rather than assuming them

- No need for explicit weather features

Note on June 30 results: some models (RandomForest, ExtraTrees, both naive baselines) show 0.0 MAE. That's impossible on real solar data; it points to a data-leakage issue in that specific test split. Under investigation. The 41-day aggregate results are what we trust.

## 6. Key Findings

| Finding | Implication |
| --- | --- |
| Data volume >> model choice | 41 days enables 0.93 W; 14 days gives 10.5 W. Collect more data first. |
| LSTM 4× better than Prophet | Use LSTM for scheduling. |
| 23% avg camera duty realistic | Not 100% uptime; battery constraint binds. |
| Battery safety maintained | Current strategy never violates 20 to 90% SoC. |
| K_SCALE = 0.11 unvalidated | Next: compare Open-Meteo forecast to actual panel_w. |

## 7. Deliverables & Next Steps

Completed:

- ✓ 35-day data cleaned and validated

- ✓ LSTM trained and compared against Prophet + 11 other models

- ✓ Optimization framework (proportional allocation)

- ✓ Input validation against real telemetry

- ✓ Fractional MAE analysis (realistic constraints)

Pending (post-feedback):

- ☐ Daily refresh loop (pull forecast each morning)

- ☐ Battery SoC integration (read voltage, calculate Wh)

- ☐ Field validation (compare scheduled vs actual uptime)

- ☐ K_SCALE validation (forecast vs real data)

## Discussion Points

- Should we prioritize camera uptime or battery longevity?

- Is daily refresh sufficient, or is hourly tuning needed?

- When should we field-test the scheduler?