IMTA Solar-Powered Buoy — Progress Report

Device 0a10aced  ·  3-week summary  ·  data live since May 20, 2026

1.  Data Collection — Live & Growing

Buoy streaming continuously since May 20. Pulled at three points: Jun 4 (~15 days, 114,960 records), Jun 14 (~25 days), Jun 24 (~35 days). Key fields: panel_w (solar), batt_v (battery), uwcam / topcams (camera relay state).

2.  Forecasting Benchmark — 11 Models, 3 Datasets

| Model (solar panel_w) | Jun 4 | Jun 14 | Jun 24 |
| --- | --- | --- | --- |
| Naive Seasonal (24h) | 5.06 W | 0.72 W ✅ | 4.04 W |
| SVR | 5.87 W | 3.43 W | 4.14 W ✅ |
| RandomForest | 6.24 W | 2.55 W | 5.27 W |
| ExtraTrees | 6.20 W | 2.74 W | 4.68 W |

Naive Seasonal peaked at 0.72 W on Jun 14 — nearly perfect. By Jun 24 weather grew more variable and SVR edged ahead (4.14 W vs 4.04 W); ML is starting to close the gap. Battery batt_v: Naive Seasonal MAE = 0.009 V — essentially perfect, driven by a stable daily charge/discharge cycle.

3.  IMTA Solar Data vs. Historical Weather

Compared the buoy's hourly panel_w to ERA5 solar-radiation data (European weather service) for the buoy's GPS location. They track each other closely during daylight hours, and the comparison estimates how much available sunlight the panel converts — the conversion factor needed to forecast from weather data. (Exact correlation pending plot review.)

4.  First Working Camera Scheduler (in progress)

A working first version: (1) pulls a free 7-day Open-Meteo forecast for the location, (2) converts it to expected solar power via the ERA5 factor above, (3) runs an optimization choosing which hours to run cameras — maximizing on-time while keeping the battery above a safe minimum. Test run today: ~107 W peak forecast tomorrow; scheduler chose 29 of the next 48 hours — all daylight, off at night. (Depends on battery size and camera draw — see open questions.)

5.  Literature Review — In Progress

10 of 22 papers read. Looking for prior work combining solar forecasting + smart scheduling + marine monitoring — so far none exists; existing papers cover one piece, not the full stack. That gap is the research contribution. Covered so far: deep learning for battery-state estimation, RL for off-grid power scheduling, and a duty-cycle optimizer for a similar low-power IoT sensor (most directly applicable to IMTA).

Open Questions for Paul

- Battery spec — actual Wh capacity and min/max voltage bounds?

- Camera draw — measured power consumption for uwcam and topcams?

- Target uptime — what % camera-on time is the project goal?

- Sensor sync — do uwcam and topcams switch independently or always together?

- Next data source — is IMTA video available for computer-vision work?

What's Next

- Add Open-Meteo forecast as future_covariates in the darts ML models

- Calibrate K_SCALE and battery parameters once confirmed with Paul

- CNN / GRU / TCN models once data > 60 days (~mid-July)

- Continue lit review (12 papers remaining)