# Notes

May 28th, 2026

IMTA System Architecture & Data Access

## Firestore Database

Project: marinesitu-imta

Database: default

### Collection: imta_telemetry

Remote data collection. Each document represents a telemetry reading.

| Field | Notes |
| --- | --- |
| timestamp | When the reading was recorded |
| device_id | Jetson identifier, assigned/tracked through Particle |
| event_type | "solar_data" or "relay_status" |
| message_id | Pub/Sub message ID — used for deduplication |
| data | Nested payload — fields vary by event_type (see below) |

#### data fields for solar_data events

| Field | Type | Notes |
| --- | --- | --- |
| batt_v | float | Battery voltage (volts) |
| batt_ma | int | Battery current (milliamps; negative = discharging) |
| panel_w | int | Solar panel power (watts) |
| state | int | Charger state: 0=Off, 2=Fault, 3=Bulk, 4=Absorption, 5=Float |
| yield_wh | int | Today's solar yield (watt-hours) |
| fw_ver | str | Firmware version (e.g. "1.0.3") |
| ve_error | int | VE.Direct error code (0 = OK) |
| lat / lon | float | GPS location (optional) |
| env_temp | float | Enclosure temperature °C (optional) |
| env_hum | float | Enclosure relative humidity % (optional) |
| env_press | float | Barometric pressure hPa (optional) |

#### data fields for relay_status events

| Field | Type | Notes |
| --- | --- | --- |
| uwcam | bool | Underwater camera power state |
| topcams | bool | Top cameras power state |
| cell_router | bool | Cellular router (device-managed) |
| keep_alive | bool | Prevents duty-cycle sleep when true |
| shutdown_pending | bool | Device shutdown is pending |
| ovr_uw / ovr_top | int | Override state: 0=auto, 1=forced on, -1=forced off |

### Collection: imta_devices

Stores current device status, relay states, and latest telemetry snapshot. (Contents to confirm)

## Hardware Architecture

- Jetson — communicates via the Particle platform

  - One or more control boxes, each containing sensors

Layout: Jetson → Control box(es) → Sensors (nested in each control box)

Dashboard note: All deployed Jetson devices expose the same parameters — unique IDs are distinct, but the parameter set is shared across devices.

Known device ID: 0a10aced202194944a071780

## Two Data Access Paths

| Option A — Query via API | Option B — Query Firestore directly ✓ chosen |
| --- | --- |
| Hit the existing API endpoints from the imta-dashboard repo. Abstracted, structured responses. Requires Stytch JWT auth token. | Use google-cloud-firestore Python SDK with Application Default Credentials (ADC). No API token needed — ADC already configured on this machine. |

## API Structure (imta-dashboard repo)

Layout under api/:

- routers/telemetry.py — endpoint logic for telemetry

  - GET /telemetry/{device_id} — latest reading

  - GET /telemetry/{device_id}/history — paginated history (cursor-based)

- routers/devices.py — endpoint logic for devices

  - GET /devices — list all devices

  - GET /devices/{device_id} — single device

- schemas/telemetry.py — SolarData, TelemetryDocument, TelemetryHistoryResponse

- schemas/devices.py — RelayState, Device, ScheduleConfig, etc.

## Action Items

- Corey — data verification

- Aditi — clone & explore the imta-dashboard repo (read telemetry.py and devices.py to understand the schema)  ✓ done

- Aditi — draft query script (with Claude)  ✓ done — see fetch_imta_data.py

- Next — run fetch_imta_data.py and verify data with Corey